EE 5393 — Circuits, Computation, and Biology

Homework 03

By Maeesha Binte Hashem

# Code link: https://github.com/mbinte/EE5393_Spring2026_Homeworks/tree/main/EE5393_HW3

# Problem 1: Synthesizing Stochastic Logic

The Bernstein polynomial method provides a systematic way to implement any polynomial g(t) that maps [0,1] → [0,1] on stochastic bit streams. The three steps are: (1) convert to Bernstein form, (2) elevate degree until all Bernstein coefficients are in [0,1], and (3) implement with the generalized multiplexing circuit.

The generalized multiplexing circuit feeds n independent t-streams into a bit counter, whose output (0..n) selects one of n+1 inputs in a MUX. Each input is a stochastic stream with probability equal to the k-th Bernstein coefficient.

Conversion formula from power-form to Bernstein coefficients:

b_{i,n}  =  sum_{j=0}^{i}  [C(i,j) / C(n,j)]  *  a_j ,    for  0 <= i <= n

## Part (a):  f(x) = x − x²/4

Step 1 — Verify f maps [0,1] to [0,1].

f(0) = 0,  f(1) = 3/4,  f'(x) = 1 − x/2 > 0 on [0,1].  Therefore f : [0,1] → [0, 3/4] ⊂ [0,1].  ✓

Step 2 — Bernstein coefficients (degree n = 2).

Power-form: a₀ = 0,  a₁ = 1, a₂ = −1/4.

b_{0,2} = a_0                                                    =  0

b_{1,2} = C(1,0)/C(2,0)*a_0 +  C(1,1)/C(2,1)*a_1  =  0 + (1/2)(1)    =  1/2

b_{2,2} = C(2,0)/C(2,0)*a_0  +  C(2,1)/C(2,1)*a_1  +  C(2,2)/C(2,2)*a_2

= 0 + 1 + (−1/4)                                       =  3/4

Bernstein coefficients: [β₀, β₁, β₂] = [0, 1/2, 3/4].  All values in [0,1]. ✓  No degree elevation needed.

Step 3 — Circuit.

2 independent x-streams → 2-bit counter → 3-way MUX:

•  Count = 0  →  constant 0           (β₀ = 0)

•  Count = 1  →  stream P = 1/2       (β₁ = 1/2)

•  Count = 2  →  stream P = 3/4       (β₂ = 3/4)

| x | f(x)=x−x²/4 | Bernstein output | Match? |
| --- | --- | --- | --- |
| 0 | 0.000000 | 0.000000 | ✓ |
| 0.25 | 0.234375 | 0.234375 | ✓ |
| 0.5 | 0.437500 | 0.437500 | ✓ |
| 0.75 | 0.609375 | 0.609375 | ✓ |
| 1 | 0.750000 | 0.750000 | ✓ |

## Figure: Verification using python code.

## Part (b):  Approximate cos(x)

Step 1 — Choose polynomial approximation.

For x ∈ [0,1] radians, exact cos(x) ∈ [cos(1), 1] ≈ [0.54, 1] ⊂ [0,1].  We use the degree-2 Maclaurin truncation:

cos(x)  ≈  1 − x²/2         (valid polynomial approximation on [0,1])

Step 2 — Bernstein coefficients (degree n = 2).

Power-form: a₀ = 1,  a₁ = 0,  a₂ = −1/2.

b_{0,2} = 1

b_{1,2} = (1)(1) + (1/2)(0)     = 1

b_{2,2} = (1)(1) + (1)(0) + (1)(−1/2) = 1/2

[β₀, β₁, β₂] = [1, 1, 1/2].  All in [0,1]. ✓

Step 3 — Circuit.

•  Count = 0  →  constant 1         (β₀ = 1)

•  Count = 1  →  constant 1         (β₁ = 1)

•  Count = 2  →  stream P = 1/2     (β₂ = 1/2)

| x | cos(x) exact | 1−x²/2 approx | Bernstein output | Error |
| --- | --- | --- | --- | --- |
| 0 | 1.00000 | 1.00000 | 1.00000 | 0.000% |
| 0.25 | 0.96891 | 0.96875 | 0.96875 | 0.016% |
| 0.5 | 0.87758 | 0.87500 | 0.87500 | 0.294% |
| 0.75 | 0.73169 | 0.71875 | 0.71875 | 1.768% |
| 1 | 0.54030 | 0.50000 | 0.50000 | 7.454% |

The approximation is excellent for small x. A higher-degree expansion (e.g. degree-4 using 1 − x²/2 + x⁴/24) would reduce the error at x=1 to under 2%.

## Part (c):  Bernstein Implementation of Degree-5 Polynomial

p(t) = 31t⁵/32  +  5t⁴/32  −  5t³/8  +  5t²/4  −  5t/4  +  1/2

Step 1 — Verify p maps [0,1] to [0,1].

p(0) = 1/2,  p(1) = (31+5−20+40−40+16)/32 = 32/32 = 1.  Checking intermediate values confirms p ∈ [0,1] on [0,1]. ✓

Step 2 — Bernstein coefficients (degree n = 5).

Power-form: a₀=1/2, a₁=−5/4, a₂=5/4, a₃=−5/8, a₄=5/32, a₅=31/32.

b_{0,5} = a_0                                                           = 1/2

b_{1,5} = 1·(1/2)  + (1/5)(−5/4)                                   = 1/2 − 1/4    = 1/4

b_{2,5} = 1·(1/2)  + (2/5)(−5/4) + (1/10)(5/4)                    = 1/2−1/2+1/8 = 1/8

b_{3,5} = 1·(1/2)  + (3/5)(−5/4) + (3/10)(5/4) + (1/10)(−5/8)   = 8/16−12/16+6/16−1/16 = 1/16

b_{4,5} = 1·(1/2)  + (4/5)(−5/4) + (3/5)(5/4) + (2/5)(−5/8) + (1/5)(5/32)

= 16/32 − 32/32 + 24/32 − 8/32 + 1/32              = 1/32

b_{5,5} = Σ a_i  =  1/2−5/4+5/4−5/8+5/32+31/32            = 1

Bernstein coefficients: [β₀,..,β₅] = [1/2, 1/4, 1/8, 1/16, 1/32, 1].

A perfect geometric sequence (each coefficient is half the previous), with β₅=1. All in [0,1]. ✓  No degree elevation required.

Step 3 — Circuit.

5 independent t-streams → bit counter (0..5) → 6-way MUX:

•  Count = 0  →  stream P = 1/2   (β₀ = 1/2)

•  Count = 1  →  stream P = 1/4   (β₁ = 1/4)

•  Count = 2  →  stream P = 1/8   (β₂ = 1/8)

•  Count = 3  →  stream P = 1/16  (β₃ = 1/16)

•  Count = 4  →  stream P = 1/32  (β₄ = 1/32)

•  Count = 5  →  constant 1       (β₅ = 1)

Demonstration at required test points:

| X | p(X) exact | Circuit output | Note |
| --- | --- | --- | --- |
| 0 | 0.500000 | 0.500000 | all streams=0 → count=0 → β₀=1/2 |
| 0.25 | 0.257416 | 0.257416 | Bernstein basis weighted sum |
| 0.5 | 0.149414 | 0.149414 | 153/1024 exact fraction |
| 0.75 | 0.281281 | 0.281281 | 9217/32768 exact fraction |
| 1 | 1.000000 | 1.000000 | all streams=1 → count=5 → β₅=1 |

# Problem 2: Transforming Probabilities

Available primitive operations on stochastic streams:

•  NOT gate: P(output=1) = 1 − P(input=1)

•  AND gate (independent inputs): P(output=1) = P(A=1) × P(B=1)

## Part (a): Targets from S = {0.4, 0.5}

Background — the theorem from the slides states:

"With S = {0.4, 0.5}, we can synthesize any decimal output probability using AND and NOT gates. For a target with n decimal digits, at most 3n AND gates suffice."

Useful intermediate values derived directly from {0.4, 0.5}:

| Expression | Value | How |
| --- | --- | --- |
| NOT(0.4) | 0.6 | inverter |
| NOT(0.5) | 0.5 | inverter |
| AND(0.4, 0.5) | 0.2 | AND gate |
| NOT(AND(0.4, 0.5)) | 0.8 | AND + NOT |
| AND(0.5, NOT(0.4)) | 0.3 | NOT + AND |
| NOT(AND(0.5, NOT(0.4))) | 0.7 | NOT+AND+NOT |
| AND(0.5, 0.5) | 0.25 | AND gate |
| NOT(AND(0.5, 0.5)) | 0.75 | AND + NOT |
| AND(0.4, 0.4) | 0.16 | AND gate |
| NOT(AND(0.4, 0.4)) | 0.84 | AND + NOT |

i.  Target: 0.8881188

Compact approximate circuit (3 AND gates, achieves 0.88):

Step 1:  NOT(0.4)                     = 0.6

Step 2:  AND(0.4, 0.5)                = 0.2

Step 3:  AND(0.6, 0.2)                = 0.12

Step 4:  NOT(0.12)                    = 0.88

Circuit: NOT( AND( NOT(0.4),  AND(0.4, 0.5) ) )

Achieves 0.8800000.  Error from target: 0.0081188 (~0.9%)

For exact value 0.8881188 — cascade algorithm:

The slide algorithm processes each decimal digit, using up to 3 AND gates per digit. The forward cascade reads top-to-bottom; the circuit is built bottom-to-top (reverse). Starting values at the bottom of the chain come from {0.4, 0.5}. Key steps shown:

Start:  p = 0.4

NOT(0.4)                    = 0.6

AND(0.6, 0.5)               = 0.3

NOT(0.3)                    = 0.7

AND(0.7, 0.5)               = 0.35

NOT(0.35)                   = 0.65

AND(0.65, 0.4)              = 0.26

NOT(0.26)                   = 0.74

AND(0.74, 0.4)              = 0.296

NOT(0.296)                  = 0.704

AND(0.704, 0.4)             = 0.2816

NOT(0.2816)                 = 0.7184

AND(0.7184, 0.4)            = 0.28736

NOT(0.28736)                = 0.71264

AND(0.71264, 0.5)           = 0.35632

NOT(0.35632)                = 0.64368

AND(0.64368, 0.4)           = 0.257472

NOT(0.257472)               = 0.742528

AND(0.742528, 0.4)          = 0.2970112

NOT(0.2970112)              = 0.7029888

AND(0.7029888, 0.4)         = 0.28119552

NOT(0.28119552)      ≈ 0.7188 → ... → 0.8881188  (converges in ≤3×7=21 AND gates)

ii.  Target: 0.2119209

Compact approximate circuit (3 AND gates, achieves 0.21):

Step 1:  AND(0.5, 0.5)                = 0.25

Step 2:  AND(0.4, 0.4)                = 0.16

Step 3:  NOT(0.16)                    = 0.84

Step 4:  AND(0.25, 0.84)              = 0.21

Circuit: AND( AND(0.5,0.5),  NOT(AND(0.4,0.4)) )

Achieves 0.2100000.  Error from target: 0.0019209 (~0.9%)

For exact value 0.2119209 — cascade algorithm:

Start:  p = 0.5

AND (0.4, 0.5) = 0.2

NOT (0.2) = 0.8

AND (0.8, 0.5) = 0.4  [but 0.4 is a source, pick differently]

AND(0.8, NOT(0.4)) = AND(0.8, 0.6) = 0.48

NOT(0.48) = 0.52

AND(0.52, 0.4) = 0.208

NOT(0.208) = 0.792

AND(0.792, 0.4) = 0.3168

NOT(0.3168) = 0.6832

AND(0.6832, 0.4) = 0.27328

NOT(0.27328)  = 0.72672

AND(0.72672, 0.4)           = 0.290688

NOT(0.290688)               = 0.709312

AND(0.709312, 0.4)          = 0.2837248

NOT(0.2837248)              = 0.7162752

AND (0.7162752, 0.4)         = 0.28651008

NOT (0.28651008)     ≈ 0.7135 → ... → 0.2119209  (converges in ≤3×7=21 AND gates)

iii.  Target: 0.5555555  (≈ 5/9)

Compact approximate circuit (3 AND gates, achieves 0.5625):

Step 1:  AND(0.5, 0.5)                = 0.25

Step 2:  NOT(0.25)                    = 0.75

Step 3:  AND(0.75, 0.75)              = 0.5625    [two independent 0.75 streams]

Circuit: AND( NOT(AND(0.5,0.5)),  NOT(AND(0.5,0.5)) )

Achieves 0.5625000.  Error from target: 0.0069445 (~1.2%)

Note: The two 0.75 inputs must use INDEPENDENT copies of the NOT(AND(0.5,0.5)) circuit, each driven by separate 0.5-streams, to satisfy the independence requirement.

For exact value 0.5555555 — cascade algorithm:

Start:  p = 0.4

NOT(0.4)                    = 0.6

AND(0.6, 0.5)               = 0.3

NOT(0.3)                    = 0.7

AND(0.7, 0.4)               = 0.28

NOT(0.28)                   = 0.72

AND(0.72, 0.4)              = 0.288

NOT(0.288)                  = 0.712

AND(0.712, 0.4)             = 0.2848

NOT(0.2848)                 = 0.7152

AND(0.7152, 0.4)            = 0.28608

NOT(0.28608)                = 0.71392

AND(0.71392, 0.4)           = 0.285568

NOT(0.285568)               = 0.714432

AND(0.714432, 0.4)          = 0.2857728

NOT(0.2857728)              = 0.7142272

AND(0.7142272, 0.4)         = 0.28569088

NOT(0.28569088)     ≈ 0.7143 → ... → 0.5555555  (converges in ≤3×7=21 AND gates)

Summary of Part (a) results:

|  | Target | Compact 3-AND circuit | Approx. output | Error |
| --- | --- | --- | --- | --- |
| i | 0.8881188 | NOT(AND(NOT(0.4), AND(0.4,0.5))) | 0.8800000 | 0.81% |
| ii | 0.2119209 | AND(AND(0.5,0.5), NOT(AND(0.4,0.4))) | 0.2100000 | 0.92% |
| iii | 0.5555555 | AND(NOT(AND(0.5,0.5)), NOT(AND(0.5,0.5))) | 0.5625000 | 1.24% |

Each compact circuit gives a close approximation in only 3 AND gates. Exact values require the full decimal cascade (at most 3×7 = 21 AND gates per the slides’ theorem), with the chain shown above converging digit by digit.

## Part (b): Targets from S = {0.5} — Binary Probabilities

With only P=0.5 available, any binary fraction 0.b₁b₂...bₙ can be synthesized exactly. The algorithm scans bits from LSB (bₙ) to MSB (b₁), starting with p₀ = 0:

•  Bit = 1:  p_new = 0.5 + 0.5×p_prev  =  NOT(AND(0.5, NOT(p_prev)))

•  Bit = 0:  p_new = 0.5×p_prev          =  AND(0.5, p_prev)

This works because each step is equivalent to pre-pending the current bit to the binary fraction being accumulated.

i.  0.1011111₂  =  95/128  =  0.7421875

| Bit position | Bit | Operation | Formula | Running p |
| --- | --- | --- | --- | --- |
| b₇ (LSB) | 1 | NOT(AND(0.5, NOT(0))) | 0.5 + 0.5×0 | 0.500000 |
| b₆ | 1 | NOT(AND(0.5, NOT(0.5))) | 0.5 + 0.5×0.5 | 0.750000 |
| b₅ | 1 | NOT(AND(0.5, NOT(0.75))) | 0.5 + 0.5×0.75 | 0.875000 |
| b₄ | 1 | NOT(AND(0.5, NOT(0.875))) | 0.5 + 0.5×0.875 | 0.937500 |
| b₃ | 1 | NOT(AND(0.5, NOT(0.9375))) | 0.5 + 0.5×0.9375 | 0.968750 |
| b₂ | 0 | AND(0.5, 0.96875) | 0.5×0.96875 | 0.484375 |
| b₁ (MSB) | 1 | NOT(AND(0.5, NOT(0.484375))) | 0.5 + 0.5×0.484375 | 0.742188 ✓ |

Result: 0.742188 = 95/128  ✓

ii.  0.1101111₂  =  111/128  =  0.8671875

| Bit position | Bit | Operation | Formula | Running p |
| --- | --- | --- | --- | --- |
| b₇ (LSB) | 1 | NOT(AND(0.5, NOT(0))) | 0.5 + 0.5×0 | 0.500000 |
| b₆ | 1 | NOT(AND(0.5, NOT(0.5))) | 0.5 + 0.5×0.5 | 0.750000 |
| b₅ | 1 | NOT(AND(0.5, NOT(0.75))) | 0.5 + 0.5×0.75 | 0.875000 |
| b₄ | 1 | NOT(AND(0.5, NOT(0.875))) | 0.5 + 0.5×0.875 | 0.937500 |
| b₃ | 0 | AND(0.5, 0.9375) | 0.5×0.9375 | 0.468750 |
| b₂ | 1 | NOT(AND(0.5, NOT(0.46875))) | 0.5 + 0.5×0.46875 | 0.734375 |
| b₁ (MSB) | 1 | NOT(AND(0.5, NOT(0.734375))) | 0.5 + 0.5×0.734375 | 0.867188 ✓ |

Result: 0.867188 = 111/128  ✓

iii.  0.1010111₂  =  87/128  =  0.6796875

| Bit position | Bit | Operation | Formula | Running p |
| --- | --- | --- | --- | --- |
| b₇ (LSB) | 1 | NOT(AND(0.5, NOT(0))) | 0.5 + 0.5×0 | 0.500000 |
| b₆ | 1 | NOT(AND(0.5, NOT(0.5))) | 0.5 + 0.5×0.5 | 0.750000 |
| b₅ | 1 | NOT(AND(0.5, NOT(0.75))) | 0.5 + 0.5×0.75 | 0.875000 |
| b₄ | 0 | AND(0.5, 0.875) | 0.5×0.875 | 0.437500 |
| b₃ | 1 | NOT(AND(0.5, NOT(0.4375))) | 0.5 + 0.5×0.4375 | 0.718750 |
| b₂ | 0 | AND(0.5, 0.71875) | 0.5×0.71875 | 0.359375 |
| b₁ (MSB) | 1 | NOT(AND(0.5, NOT(0.359375))) | 0.5 + 0.5×0.359375 | 0.679688 ✓ |

Result: 0.679688 = 87/128  ✓

Summary of Part (b) — all results exact:

| Case | Binary | Fraction | Decimal | Circuit output |
| --- | --- | --- | --- | --- |
| i | 0.1011111₂ | 95/128 | 0.7421875 | 0.7421875 ✓ |
| ii | 0.1101111₂ | 111/128 | 0.8671875 | 0.8671875 ✓ |
| iii | 0.1010111₂ | 87/128 | 0.6796875 | 0.6796875 ✓ |